# Purpose

This document explains how to use a Smartphone as a video camera for ProPresenter.

| # | DESCRIPTION | SCREENSHOT |
| --- | --- | --- |
| 1 | Download EPOCCAM from this link. Install the software. This will allow connect your phone to ProPresenter. |  |
| 2 | On your Smartphone or Tablet, download EpocCam or EpocCam Pro from the iOS store or Android store |  |
| 3 | You will need to be on the same WiFi or wired network to connect to ProPresenter |  |
| 4 | ProPresenter Go to Video Input > From Video Input > Video Input Configuration… |  |
| 5 | Click the “+” sign next to ‘VIDEO’ to add another video input or use one of the existing video inputs. If this is a first time set up, you can use one of the default 3 video inputs. |  |
| 6 | Click on ‘Device’ and choose ‘Elgato Virtual Camera’. |  |
| 7 | Make sure that the EpocCam or EpocCam Pro app is running on the Smartphone and that it is on the same network as the PC/Mac running ProPresenter. |  |
| 8 | Run Elgato Camera Hub. If the app on the Smartphone is NOT on, you will see the message to the right. |  |
| 9 | Once you start the app on the smart device, you will see this screen with whatever the forward-facing camera is pointing to. NOTE: If you are using the FREE version, you will have a watermark on the video and a some of the options will be greyed out. |  |
| 10 | You can now add this stream into a Presentation slide. Check ‘Fill’ Select ‘Video Input’ Select the Input # Resize the area for the video input |  |
| 11 | You will see the video output when you go back to ‘Show’ |  |
|  | RECASTING BACK INTO PROPRESENTER |  |
| 12 | Make the following connections: - Connect a 2nd PC/Mac into the ATEM as a camera. - Connect the USB OUT from the ATEM to the ProPresenter Mac | See Visio diagram for more information |
| 13 | After setting up the ATEM, you can: - Cast from the camera output, into a ProPresenter slide and have that go out as the feed into Facebook Live or other streaming service. - Cast directly to Facebook, bypassing ProPresenter altogether. | See this link for the diagram. |